PHB04B DLL Programmers manual

Version: v1.0

The PHB04B.dll and this manual is copyright by Chengdu xhc technology originally .This document and the DLL interface may be distributed freely as long as no modifications are made to neither one of them.

Homepage: www.cdxhctech.com

table of Contents

1.Overview

2.programming instructions

3.the function interface specification

4.error code

5.examples

## 1.Applications

We provide PHB04B.DLL to satisfy users who want to use our wireless pendant and their CNC System is based on computer.

Limitations: although the PHB04B.DLL could be embedded in many platforms, but only the CNC system with USB port could use the DLL. we advice you to use the DLL in windows platforms, we don’t assure that it could be run on the other system.

CNC System

PHB04B.DLL

PHB04B Pendant

PHB04B.DLL currently only supports Windows platforms.

PHB04B.DLL will build up the communication with PHB04B controller When the CNC system has loaded the phb04B.dll.the controller will auto connect the PHB04B pendant when the controller is inserted in the USB port. The pendant will send the key to the controller when the user is pressing, the PHB04B.dll will receive and transmit the key to the CNC System, the system will download some information to the pendant display through the DLL. So the CNC system and our PHB04B pendant builds up mutual communication through the PHB04B.dll.

## 2.How to use the dll

To use the PHB04B.DLL，please do as following：

        - Load the PHB04B.DLL and locate the exported function.

        - Call XOpen function to open the usb controller.

        - Call XSendOutput function to download information.

        - Call XGetInput function to receive keys from the pendant.

        - Call XClose function to close the usb device.

## DLL Exported functions

    - Void Xinit（void）

Initialize function,make sure that it is called first.

    - long XOpen (HWND hwndwnd)

Open usb controller device,to communicate you must open the usb device.

Parameter:

hwndwnd:the handle of the parent window,when user press the key,the key information message will be sent to the window’s handle.you could refer to our demo.

Return 0:success

Return not 0:refer to error code

    - long XSendOutput (char *pBuff, char *pSize)

download information to the device function

Parameter:

pBuff: The buffer of download lcd data,which includes 4 lines lcd data,total 59 ascii characters.

pSize: The download lcd data length.

Return not zero:error.

Return 0:success

Example:

XHC  10X      first line,maximum 11 charaters

1234567890    second line, maximum 16 charaters

Abcdefghigkk    third line, maximum 16 charaters

,.;’[]{}\|+-       fourth line, maximum 16 charaters

Every line You must fill the blanks with ‘  ’ data to the line maximum number,

    - long XGetInput (int * pHeader,char * pCount)

Read data from the device function

Parameter:

pHeader: Read data buffer pointer

pCount:  Read data length pointer

Return 0: success

Return not zero:error

The device datum are arranged like this:

Vkcode1+vkcode2+ DR+BAND+WHEEL

5 bytes：key1 data +key2 data + Ratio data+Axis data +MPG data

the gap time to read key should be less than 100ms,you could ignore this function if get key through windows message .

Example:

0x01 0x00 0x0D 0x11 0xff

The 01 key pressed，Ratio data： 1；Axis data：X Axis；MPG data：-1

0x00 0x00 00x0D 0x11 0x00

No key pressed

    - long XClose ()

Close device function

Return 0:success

Return not 0:error

4. ERROR CODE

Error code define:

100:     USB device is not open

101:     USB download error

102:     USB read error

103:     Parameter error

5． Dll exmaple

how to use our DLL,We provide a vc demo with source ,other language  could refer to it also.