PHB04B.exe Application Notes

Chengdu XingheCheng Technology Co., Ltd.

Overview：PHB04BApp.exe is our remoter’s  application Demo, This demo show that the remoter transfer the key to the app and the app download the display data to remoter. you could link  the DLL into your CNC system through this demo. then your CNC system will support our  remoter.

Open an application PHB04B.exe:

NC remote control key data

LCD display data, a total of 4 lines

Rate select

Axis select

Mpg pulse

After setting, point download

Test Procedure:

- Insert the USB receiver.

- Open an application PHB04B.exe

- Press the remote control button, the PHB04B.exe will automatically display the pressed key support 13 keys; X, Y, Z, A, B, C six-axis; Ratio data；100PPR MPG data；

- You can freely define LCD data, numbers and letters to support standards such as ASCII display. You can define maximum 59 ASCII characters. Write LCD LCD data edit box, and then download, PHB04B LCD will display the corresponding data.

- LCD data: The first line shows 11 characters, in addition to three lines of 16 characters, all ASCII codes. Supports 32-127 standard ASCII code. Does not support extended ASCII codes and Chinese display.

Chengdu XingheCheng Technology Co., Ltd.

2016-01-18